----------------------------------------------------------------------

DOCUMENT:

PROGRAMMING DIFFERENCES BETWEEN SOUND MASTER II AND SOUND BLASTER

FOR RECORDING AND PLAYBACK OF VOICE (DMA AND NON-DMA MODES)

Version 1.0

April 2, 1991

Programming and Documentation by Ryan Hanlon

Copyright (c) 1991,  Covox, Inc.  All Rights Reserved

----------------------------------------------------------------------

QUICK COMPARISON

---------------------------------------------------------

SOUND MASTER II             SOUND BLASTER

---------------------------------------------------------

BASE PORTS   0x0220, 0x0240,             0x0210, 0x0220, 0x0230

0x0280, 0x02C0              0x0240, 0x0250, 0x0260

---------------------------------------------------------

BASE PORT    8254 TIMER 0  = 0x08        RESET                 = 0x06

OFFSET       8254 TIMER 1  = 0x09        READ DATA             = 0x0A

8254 TIMER 2  = 0x0A        WRITE COMMAND/DATA    = 0x0C

8254 CONTROL  = 0x0B        WRITE BUFFER STATUS   = 0x0C

CLEAR IRQ     = 0x0C        DATA AVAILABLE STATUS = 0x0E

DISABLE VMDMA = 0x0D

ENABLE VMDMA  = 0x0E

DAC OFFSET    = 0x0F

---------------------------------------------------------

MAXIMUM      INPUT  (ADC) 25KHz          INPUT  (ADC) 13KHz

I/O RATES    OUTPUT (DAC) 100KHz         OUTPUT (DAC) 13KHz

(8 bit only)

---------------------------------------------------------

DATA         2 bit, 3 bit, 4 bit         2 bit, 3 bit, 4 bit

true ADPCM (high quality)   differential PCM (not ADPCM)

COMPRESSION  optional silence encoding   optional silence encoding

---------------------------------------------------------

AVAILABLE    3, 4, 5, 6, 7               2, 3, 5, 7

IRQ'S

---------------------------------------------------------

COMMANDS     N/A                         8 BIT DAC OUTPUT    = 0x10

8 BIT ADC INPUT     = 0x20

8 BIT DMA INPUT     = 0x24

8 BIT DMA OUTPUT    = 0x14

2 BIT DMA OUTPUT    = 0x16

2 BIT DMA REFERENCE = 0x17

2.6 BIT DMA OUTPUT  = 0x76

2.6 BIT DMA REF.    = 0x77

4 BIT DMA OUTPUT    = 0x74

4 BIT DMA REFERENCE = 0x75

HALT DMA            = 0xD0

CONTINUE DMA        = 0xD4

SET TIME CONSTANT   = 0x40

MULTI-CHANNEL DAC   = 0x6?

TURN ON SPEAKER     = 0xD1

TURN OFF SPEAKER    = 0xD3

CARD IDENTIFICATION = 0xE0

GET DSP VERSION NUM = 0xE1

----------------------------------------------------------------------

NON-DMA INPUT OF 8 BIT PCM SOUND DATA

SOUND MASTER II                  SOUND BLASTER

---------------------------      --------------------------------

INITIALIZE SM II FOR INPUT       INITIALIZE THE DSP FOR INPUT OF PCM DATA

1. Set up timing method.         1. Reset the DSP chip by writing a '1'

to the RESET PORT (offset 0x06).

2. Wait 3 micro-seconds.

3. Write a '0' to the RESET PORT.

4. Insure that the DSP was initialized

by polling for '0xAA' at the READ

DATA PORT (offset 0x0A)

5. Wait for the MSB of the WRITE DATA

COMMAND PORT (offset 0x0C) to equal

0.

6. Write 0xD3 command (TURN OFF SPEAKER)

to the WRITE DATA/COMMAND PORT.

7. Set up timing method.

INPUT PCM DATA FROM SM II        INPUT PCM DATA FROM DSP

2. Read byte of PCM data         8. Wait for the MSB of the WRITE DATA/

from DAC PORT (offset            COMMAND PORT (offset 0x0C) to equal

0x0F).                           0.

3. Wait for correct timing       9. Write 0x20 command (8 BIT ADC INPUT)

and repeat step 2 until          to the WRITE DATA/COMMAND PORT.

end of transfer.             10. Wait for the MSB of the WRITE DATA/

COMMAND PORT (offset 0x0C) to equal

1.

11. Read byte of PCM data from the READ

DATA PORT (offset 0x0A).

12. Wait for correct timing and repeat

steps 8 through 11 until end of

transfer.

----------------------------------------------------------------------

NON-DMA OUTPUT OF 8 BIT PCM SOUND DATA

SOUND MASTER II                  SOUND BLASTER

---------------------------      --------------------------------

INITIALIZE SM II FOR OUTPUT      INITIALIZE THE DSP FOR OUTPUT OF PCM DATA

1. Set up timing method.         1. Reset the DSP chip by writing a '1'

to the RESET PORT (offset 0x06).

2. Wait 3 micro-seconds.

3. Write a '0' to the RESET PORT.

4. Insure that the DSP was initialized

by polling for '0xAA' at the READ

DATA PORT (offset 0x0A)

5. Wait for the MSB of the WRITE DATA

COMMAND PORT (offset 0x0C) to equal

0.

6. Write 0xD1 command (TURN ON SPEAKER)

to the WRITE DATA/COMMAND PORT.

7. Set up timing method.

OUTPUT PCM DATA FROM SM II       INPUT PCM DATA FROM DSP

2. Read byte of PCM data         8. Wait for the MSB of the WRITE DATA/

from DAC PORT (offset            COMMAND PORT (offset 0x0C) to equal

0x0F).                           0.

3. Wait for correct timing       9. Write 0x10 command (8 BIT ADC OUTPUT)

and repeat step 2 until          to the WRITE DATA/COMMAND PORT.

end of transfer.             10. Wait for the MSB of the WRITE DATA/

COMMAND PORT (offset 0x0C) to equal 1.

11. Write a byte of PCM data to the WRITE

DATA/COMMAND PORT (offset 0x0C).

12. Wait for correct timing and repeat

steps 8 through 11 until end of

transfer.

----------------------------------------------------------------------

DMA INPUT OF 8 BIT PCM SOUND DATA

SOUND MASTER II                  SOUND BLASTER

---------------------------      --------------------------------

INITIALIZE THE SM II FOR INPUT   INITIALIZE THE DSP FOR INPUT OF PCM DATA

1. Disable VMDMA on the sm II    1. Reset the DSP chip by writing a '1'

by writting any value to         to the RESET PORT (offset 0x06).

the DISABLE VMDMA PORT        2. Wait 3 micro-seconds.

(offset 0x0D).                   3. Write a '0' to the RESET PORT.

4. Insure that the DSP was initialized

by polling for '0xAA' at the READ

DATA PORT (offset 0x0A)

5. Wait for the MSB of the WRITE DATA

COMMAND PORT (offset 0x0C) to equal

0.

6. Write 0xD3 command (TURN OFF SPEAKER)

to the WRITE DATA/COMMAND PORT.

2. Setup 8237 DMA controller     7. Setup 8237 DMA controller and IRQ

and IRQ handlers.                handlers.

3. Write control byte to the     8. Wait for the MSB of the WRITE DATA

8254 controller on the           COMMAND PORT to equal 0.

SM II.                        9. Write 0x40 command (SET TIME CONSTANT)

4. Write LSB of the input rate      to the WRITE DATA/COMMAND PORT.

to the 8254 on the SM II.    10. Wait for the MSB of the WRITE DATA

5. Write MSB of the input rate      COMMAND PORT to equal 0.

to the 8254 on the SM II.    11. Write one byte value to WRITE DATA/

COMMAND PORT for desired input rate.

6. Enable VMDMA on the SM II    12. Wait for the MSB of the WRITE DATA

by writing any value to the      COMMAND PORT to equal 0.

ENABLE VMDMA PORT (offset    13. Write 0x24 command (8 BIT DMA INPUT)

0x0E).                           to the WRITE DATA/COMMAND PORT.

----------------------------------------------------------------------

DMA OUTPUT OF 8 BIT PCM SOUND DATA

SOUND MASTER II                  SOUND BLASTER

---------------------------      --------------------------------

INITIALIZE THE SM II FOR OUTPUT  INITIALIZE THE DSP FOR OUTPUT OF PCM DATA

1. Disable VMDMA on the sm II    1. Reset the DSP chip by writing a '1'

by writting any value to         to the RESET PORT (offset 0x06).

the DISABLE VMDMA PORT        2. Wait 3 micro-seconds.

(offset 0x0D).                   3. Write a '0' to the RESET PORT.

4. Insure that the DSP was initialized

by polling for '0xAA' at the READ

DATA PORT (offset 0x0A)

5. Wait for the MSB of the WRITE DATA

COMMAND PORT (offset 0x0C) to equal

0.

6. Write 0xD2 command (TURN ON SPEAKER)

to the WRITE DATA/COMMAND PORT.

2. Setup 8237 DMA controller     7. Setup 8237 DMA controller and IRQ

and IRQ handlers.                handlers.

3. Write control byte to the     8. Wait for the MSB of the WRITE DATA

8254 controller on the           COMMAND PORT to equal 0.

SM II.                        9. Write 0x40 command (SET TIME CONSTANT)

4. Write LSB of the input rate      to the WRITE DATA/COMMAND PORT.

to the 8254 on the SM II.    10. Wait for the MSB of the WRITE DATA

5. Write MSB of the input rate      COMMAND PORT to equal 0.

to the 8254 on the SM II.    11. Write one byte value to WRITE DATA/

COMMAND PORT for desired input rate.

6. Enable VMDMA on the SM II    12. Wait for the MSB of the WRITE DATA

by writing any value to the      COMMAND PORT to equal 0.

ENABLE VMDMA PORT (offset    13. Write 0x14 command (8 BIT DMA OUTPUT)

0x0E).                           to the WRITE DATA/COMMAND PORT.